Grove Multichannel Gas Sensor V2
STM32MP157-DK1 → ESP32-A → ESP32-B

Engineering Integration & Validation Document

Document status: Milestone completed / baseline validated

Scope: Grove Gas Sensor V2 acquisition on STM32MP157-DK1, UART transport using a POSIX shell script, UART reception on XIAO ESP32-C3, and ESP-NOW transport to a second XIAO ESP32-C3.

Out of scope: CRC, retries, acknowledgements, channel hopping, jamming resistance, ppm calibration, and production hardening. These are intentionally held for a later milestone.

# 1. System Overview

This milestone implements and validates a complete gas-sensor telemetry path. The Grove Multichannel Gas Sensor V2 is connected to the Linux side of the STM32MP157D-DK1 over I²C. The STM32 Linux userspace reads three required gas channels (NO₂, Ethanol and CO), packages the raw readings into a simple ASCII record, and transmits the record through UART7. ESP32-A receives the UART record and forwards the same bytes through ESP-NOW. ESP32-B receives the ESP-NOW payload, parses it, and displays the gas names and raw sensor values.

Grove Multichannel Gas Sensor V2
          │
          │ I²C (7-bit address 0x08)
          ▼
STM32MP157D-DK1 / OpenSTLinux
          │
          │ UART7 / /dev/ttySTM2
          │ 115200 8N1
          ▼
XIAO ESP32-C3 (ESP32-A)
          │
          │ ESP-NOW
          ▼
XIAO ESP32-C3 (ESP32-B)
          │
          ▼
Human-readable gas data on Serial Monitor

# 2. Hardware Used

| Component | Role | Relevant details |
| --- | --- | --- |
| STM32MP157D-DK1 | Linux gateway / sensor host | OpenSTLinux; I²C5 + UART7 |
| Grove Multichannel Gas Sensor V2 | Gas sensing | Grove I²C; default 7-bit address 0x08 |
| XIAO ESP32-C3 – ESP32-A | UART → ESP-NOW bridge | GPIO20 RX, GPIO21 TX |
| XIAO ESP32-C3 – ESP32-B | ESP-NOW receiver / display | Serial Monitor output |

# 3. Gas Sensor Channel Mapping

| Sensor element | Gas label | Command | Telemetry |
| --- | --- | --- | --- |
| GM-102B | NO₂ | 0x01 | NO2 |
| GM-302B | Ethanol | 0x03 | Ethanol |
| GM-502B | VOC | 0x05 | Not transmitted |
| GM-702B | CO | 0x07 | CO |

Only NO₂, Ethanol and CO are included in this milestone. VOC/GM-502B is intentionally excluded.

# 4. Measurement Interpretation

The transmitted values are raw sensor readings, not ppm. The Seeed module documentation describes the module as suitable for qualitative rather than quantitative measurements. A sensor-element concentration range is not, by itself, a validated raw-output-to-ppm conversion.

# 5. STM32 I²C Integration

Linux I²C5 was initially disabled in the active device tree. Only the Linux I²C5 controller node was changed from status="disabled" to status="okay". The Cortex-M4 resource node was deliberately not changed.

- status = "disabled";
+ status = "okay";

After reboot, the controller at 0x40015000 appeared as /dev/i2c-1.

i2cdetect -l

i2c-0 i2c STM32F7 I2C(0x40012000) I2C adapter
i2c-1 i2c STM32F7 I2C(0x40015000) I2C adapter
i2c-2 i2c STM32F7 I2C(0x5c002000) I2C adapter
i2c-3 i2c i2c-0-mux (chan_id 0) I2C adapter

Live pinmux:
pin 11 (PA11): device 40015000.i2c function af4 group PA11
pin 12 (PA12): device 40015000.i2c function af4 group PA12

# 6. Grove Sensor ↔ STM32 Wiring

| Sensor | STM32MP157D-DK1 | Signal |
| --- | --- | --- |
| SCL | CN2 pin 5 / PA11 | I²C5_SCL |
| SDA | CN2 pin 3 / PA12 | I²C5_SDA |
| VCC | 3.3 V | Power |
| GND | CN2 pin 6 | Ground |

No solder-bridge changes were required for this direct CN2 connection. The working configuration was validated from the live device-tree/pinmux state.

# 7. Sensor Address and Protocol

The sensor was detected at the 7-bit I²C address 0x08.

i2cdetect -y 1

     0  1  2  3  4  5  6  7  8  9  a  b  c  d  e  f
00:                         08 -- -- -- -- -- -- --

The value 0x55 encountered during investigation was not the default sensor address; it is the command used by the Seeed V2 protocol for changing the I²C address.

Read sequence:
1. Write one command byte to 0x08
2. Wait approximately 1 ms
3. Read 4 bytes
4. Interpret the four bytes as a little-endian uint32 raw value

# 8. Direct I²C Validation

i2ctransfer -y 1 w1@0x08 0x01 r4
0xe5 0x00 0x00 0x00

i2ctransfer -y 1 w1@0x08 0x03 r4
0x41 0x00 0x00 0x00

i2ctransfer -y 1 w1@0x08 0x07 r4
0x9c 0x00 0x00 0x00

These corresponded to raw values 229, 65 and 156, confirming the sensor and Linux I²C path before UART integration. Warm-up was observed during testing; readings changed during early operation and later became comparatively stable.

# 9. STM32 UART7 Configuration

UART7 is exposed by Linux as /dev/ttySTM2. PE8 is UART7_TX and PE7 is UART7_RX.

| Signal | STM32 pin | DK1 connector | Connection |
| --- | --- | --- | --- |
| UART7_TX | PE8 | CN14 pin 2 | ESP32-A GPIO20 RX |
| UART7_RX | PE7 | CN14 pin 1 | ESP32-A GPIO21 TX |
| GND | GND | Board GND | Common ground |

stty -F /dev/ttySTM2 115200 cs8 -cstopb -parenb -ixon -ixoff -crtscts

# 10. STM32 ↔ ESP32-A Wiring

STM32MP157-DK1                    XIAO ESP32-C3 (ESP32-A)

CN14 pin 2 / PE8 / UART7_TX  ------>  GPIO20 / RX
CN14 pin 1 / PE7 / UART7_RX  <------  GPIO21 / TX
STM32 GND                    ------->  ESP32 GND
STM32 3V3                    ------->  ESP32 3V3

# 11. STM32 Gas-to-UART Bridge

A POSIX /bin/sh script was used for the prototype because i2c-tools and the UART device were already available on OpenSTLinux. The script starts sensor warming, reads NO₂/Ethanol/CO every two seconds, creates the GAS record, and writes it to /dev/ttySTM2.

#!/bin/sh

I2C_BUS=1
SENSOR_ADDR=0x08
UART_DEV=/dev/ttySTM2

stty -F "$UART_DEV" 115200 cs8 -cstopb -parenb -ixon -ixoff -crtscts

# Start sensor warming
i2cset -y "$I2C_BUS" "$SENSOR_ADDR" 0xFE

echo "Gas sensor UART bridge started."
echo "I2C bus : $I2C_BUS"
echo "Sensor  : $SENSOR_ADDR"
echo "UART    : $UART_DEV"

SEQ=1

read_sensor()
{
    CMD="$1"
    DATA=$(i2ctransfer -y "$I2C_BUS" w1@"$SENSOR_ADDR" "$CMD" r4 2>/dev/null)

    if [ $? -ne 0 ] || [ -z "$DATA" ]; then
        echo "ERROR: I2C read failed for command $CMD" >&2
        return 1
    fi

    set -- $DATA
    VALUE=$(( $1 | ($2 << 8) | ($3 << 16) | ($4 << 24) ))
    echo "$VALUE"
}

while true
do
    NO2=$(read_sensor 0x01)
    ETHANOL=$(read_sensor 0x03)
    CO=$(read_sensor 0x07)

    if [ -n "$NO2" ] && [ -n "$ETHANOL" ] && [ -n "$CO" ]; then
        MESSAGE=$(printf "GAS,%06d,%d,%d,%d" \
            "$SEQ" "$NO2" "$ETHANOL" "$CO")

        printf '%s\r\n' "$MESSAGE" > "$UART_DEV"
        echo "STM32 TX: $MESSAGE"
        SEQ=$((SEQ + 1))
    else
        echo "ERROR: Invalid sensor reading"
    fi

    sleep 2
done

# 12. Telemetry Record Format

GAS,<sequence>,<NO2_raw>,<Ethanol_raw>,<CO_raw>

Example: GAS,000006,309,64,161

| Field | Meaning |
| --- | --- |
| GAS | Packet type / record identifier |
| 000006 | Sequence number |
| 309 | NO₂ raw value |
| 64 | Ethanol raw value |
| 161 | CO raw value |

# 13. ESP32-A — UART to ESP-NOW Bridge

#include <Arduino.h>
#include <WiFi.h>
#include <esp_now.h>

HardwareSerial STM32Serial(1);

static const int STM32_RX_PIN = 20;
static const int STM32_TX_PIN = 21;

uint8_t peerAddress[] = {
    0x58, 0x8C, 0x81, 0xA9, 0xE2, 0x18
};

void onDataSent(const wifi_tx_info_t *info, esp_now_send_status_t status)
{
    Serial.print("ESP-NOW TX: ");
    Serial.println(status == ESP_NOW_SEND_SUCCESS ? "SUCCESS" : "FAIL");
}

void setup()
{
    Serial.begin(115200);
    delay(1000);

    Serial.println();
    Serial.println("================================");
    Serial.println(" STM32 UART -> ESP-NOW BRIDGE");
    Serial.println("================================");

    STM32Serial.begin(115200, SERIAL_8N1,
                      STM32_RX_PIN, STM32_TX_PIN);

    Serial.println("UART configured:");
    Serial.println("  Baud : 115200");
    Serial.println("  RX   : GPIO20");
    Serial.println("  TX   : GPIO21");

    WiFi.mode(WIFI_STA);

    Serial.print("ESP32-A MAC: ");
    Serial.println(WiFi.macAddress());

    if (esp_now_init() != ESP_OK)
    {
        Serial.println("ERROR: ESP-NOW init failed");
        return;
    }

    esp_now_register_send_cb(onDataSent);

    esp_now_peer_info_t peerInfo = {};
    memcpy(peerInfo.peer_addr, peerAddress, 6);
    peerInfo.channel = 0;
    peerInfo.encrypt = false;

    if (esp_now_add_peer(&peerInfo) != ESP_OK)
    {
        Serial.println("ERROR: Failed to add ESP32-B peer");
        return;
    }

    Serial.println("ESP-NOW peer added.");
    Serial.println("Waiting for STM32 UART data...");
}

void loop()
{
    if (STM32Serial.available())
    {
        String message = STM32Serial.readStringUntil('\n');
        message.trim();

        if (message.length() > 0)
        {
            Serial.print("UART RX: ");
            Serial.println(message);

            esp_err_t result = esp_now_send(
                peerAddress,
                (uint8_t *)message.c_str(),
                message.length()
            );

            if (result != ESP_OK)
            {
                Serial.print("ESP-NOW send error: ");
                Serial.println(result);
            }
        }
    }
}

# 14. ESP32-B — ESP-NOW Gas Receiver

#include <Arduino.h>
#include <WiFi.h>
#include <esp_now.h>

void onDataRecv(
    const esp_now_recv_info_t *info,
    const uint8_t *data,
    int len
)
{
    char packet[250];

    if (len >= sizeof(packet))
    {
        Serial.println("ERROR: Packet too large");
        return;
    }

    memcpy(packet, data, len);
    packet[len] = '\0';

    Serial.println();
    Serial.println("================================");
    Serial.println("       GAS SENSOR DATA");
    Serial.println("================================");

    Serial.print("Packet  : ");
    Serial.println(packet);

    char type[10];
    unsigned long sequence;
    int no2;
    int ethanol;
    int co;

    int fields = sscanf(
        packet,
        "%9[^,],%lu,%d,%d,%d",
        type, &sequence, &no2, &ethanol, &co
    );

    if (fields != 5)
    {
        Serial.println();
        Serial.println("STATUS  : INVALID");
        Serial.println("Reason  : Invalid packet format");
        Serial.println("================================");
        return;
    }

    if (strcmp(type, "GAS") != 0)
    {
        Serial.println();
        Serial.println("STATUS  : INVALID");
        Serial.println("Reason  : Unknown packet type");
        Serial.println("================================");
        return;
    }

    Serial.println();
    Serial.print("Sequence : ");
    Serial.println(sequence);

    Serial.println();

    Serial.print("NO2      : ");
    Serial.print(no2);
    Serial.println(" RAW");

    Serial.print("Ethanol  : ");
    Serial.print(ethanol);
    Serial.println(" RAW");

    Serial.print("CO       : ");
    Serial.print(co);
    Serial.println(" RAW");

    Serial.println();
    Serial.println("STATUS   : VALID");
    Serial.println("================================");
}

void setup()
{
    Serial.begin(115200);
    delay(1000);

    Serial.println();
    Serial.println("================================");
    Serial.println("       ESP-NOW GAS RECEIVER");
    Serial.println("================================");

    WiFi.mode(WIFI_STA);

    Serial.print("ESP32-B MAC: ");
    Serial.println(WiFi.macAddress());

    if (esp_now_init() != ESP_OK)
    {
        Serial.println("ERROR: ESP-NOW initialization failed");
        return;
    }

    esp_now_register_recv_cb(onDataRecv);

    Serial.println("ESP-NOW receiver ready.");
    Serial.println("Waiting for gas sensor data...");
}

void loop()
{
}

# 15. ESP-NOW Configuration

| Parameter | ESP32-A | ESP32-B |
| --- | --- | --- |
| Role | UART → ESP-NOW transmitter | ESP-NOW receiver |
| MAC | 94:A9:90:6A:22:60 | 58:8C:81:A9:E2:18 |
| Peer channel | 0 | Current channel |
| Encryption | Disabled | Disabled |
| Arduino ESP32 core | 3.3.11 | 3.3.11 |

# 16. Validation Evidence

STM32 → ESP32-A UART validation:

STM32 TX: GAS,000005,219,66,162
ESP32-A:  UART RX: GAS,000005,219,66,162

STM32 TX: GAS,000006,219,66,162
ESP32-A:  UART RX: GAS,000006,219,66,162

ESP32-B received an intact ESP-NOW record:

22:33:29.781 -> ESP-NOW RX: GAS,000001,216,66,161

Decoded ESP32-B output:

Sequence : 5

NO2      : 310 RAW
Ethanol  : 64 RAW
CO       : 161 RAW

STATUS   : VALID
================================

================================
       GAS SENSOR DATA
================================
Packet  : GAS,000006,309,64,161

Sequence : 6

NO2      : 309 RAW
Ethanol  : 64 RAW
CO       : 161 RAW

STATUS   : VALID
================================

# 17. Milestone Result

The milestone is validated as an end-to-end raw gas telemetry pipeline.

| Stage | Result |
| --- | --- |
| Grove Gas Sensor → STM32 I²C | PASS |
| STM32 sensor acquisition | PASS |
| STM32 UART7 → ESP32-A | PASS |
| ESP32-A UART reception | PASS |
| ESP32-A → ESP32-B ESP-NOW | PASS |
| ESP32-B packet parsing | PASS |
| Human-readable gas display | PASS |

# 18. Known Limitations / Deliberate Scope

The transmitted gas values are raw sensor values, not ppm.

No sensor calibration curve has been established.

VOC/GM-502B is not included in the current telemetry record.

STATUS: VALID means the expected text structure parsed successfully; it is not a CRC or cryptographic integrity result.

No CRC, retransmission, ACK, duplicate suppression, sequence-gap handling, channel hopping, or interference/jamming-resistance mechanism is included.

The STM32 bridge is a prototype POSIX shell implementation.

The sensor requires warm-up; readings changed during warm-up and later became comparatively stable.

# 19. Reproduction Procedure

Connect SCL→CN2 pin 5/PA11, SDA→CN2 pin 3/PA12, VCC→3.3 V, GND→CN2 pin 6.

Enable Linux I²C5 in the active device tree and verify /dev/i2c-1.

Run i2cdetect -y 1 and confirm address 0x08.

Verify direct reads with i2ctransfer for commands 0x01, 0x03 and 0x07.

Configure /dev/ttySTM2 as 115200 8N1.

Connect CN14 pin 2/PE8 to ESP32-A GPIO20 RX and CN14 pin 1/PE7 to GPIO21 TX; share ground.

Upload ESP32-A UART-to-ESP-NOW bridge.

Upload ESP32-B gas receiver.

Run /root/gas_uart_bridge.sh on STM32.

Verify STM32 TX, ESP32-A UART RX, and ESP32-B decoded output.

# 20. Reference Documents

STMicroelectronics UM2637, STM32MP157D-DK1 / STM32MP157F-DK2 Discovery kit user manual. The uploaded STM32MP157 device datasheet was used for the PE7/PE8 UART7 alternate-function assignment. Seeed Grove Multichannel Gas Sensor V2 documentation/source was used for the sensor command mapping, I²C address, and raw/qualitative measurement interpretation.

# 21. Final Data Path

GM-102B (NO2) ─┐
GM-302B (Ethanol) ─┼─> Sensor MCU / I²C 0x08
GM-702B (CO) ──────┘
                         │
                         ▼
                 STM32MP157-DK1
                    /dev/i2c-1
                         │
                         │ POSIX sh
                         │ /dev/ttySTM2
                         ▼
                    UART7 115200
                         │
                         ▼
                    ESP32-A
                 UART -> ESP-NOW
                         │
                         ▼
                    ESP32-B
                  parse + display
                         │
                         ▼
                 NO2 : <raw>
                 Ethanol : <raw>
                 CO : <raw>
                 STATUS : VALID

End of milestone documentation